CF102

Honda CB 300 R_2018-

| Kit for Left Side |  |  |  |  | Kit for Right Side |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | Description | Cat. No. | Pcs. |  | ID | Description | Cat. No. | Pcs. |
| 1 | Frame | CF102 | 1 |  | 1 | Frame | CF102 | 1 |
| 2 | Dural. roller d=36mm | 1239 | 1 |  | 2 | Dural. roller d=36mm | 1239 | 1 |
| 3 | Safety nut M10 | 810 | 3 |  | 3 | Safety nut M10 | 810 | 3 |
| 4 | Washer M10 flat | 804 | 3 |  | 4 | Washer M10 flat | 804 | 3 |
| 5 | Rod 10x285x30x30 |  | 1 |  | 5 | Dural. roller d=47mm | 48 | 1 |
| 6 | Rod 10x120x30x30 |  | 1 |  | 6 | RDmoto Stickers | L+P | 2 |
| 7 | Rod 10x230x30x30 |  | 1 |  |  |  |  |  |

We recommend using medium strenght thread glue during assembly.

Instructions (same for both sides):

- Take off the original bolts and replace them with rods, as shown on attached pictures.

- Take the ID.2 roller and fit it onto the rod in front part from both sides of the frame.

- Starting with left side, fit the ID.4 M10 washers and M10 ID.4 nuts onto all three rods in the frame.

- Then, fit the frame from right side and insert the ID.5 roller between the motorcycle and crash frame in the rear part.

- Then, take the M10 ID.4 washers, attach them using M10 ID.3 nuts and fit them onto the remaining three rods.

- Finally, tighten the nuts on both sides and make sure that all fits.

ID6 + ID2+ ID3,4

ID7 + ID3 + ID4

ID5

ID5 + ID3 + ID4

Rod 10x285 ID.5+ Dural.roller ID.2 + Washer M10 ID.4 + Nut M10 ID.3

Rod 10x120 ID.6 + Dural.roller ID.2 + Washer  M10 ID.4

Rod 10x230 + ID.7 + Dural.roller ID.2 + Washer M10 ID.4

Dural.roller ID.2 + Washer M10 ID.4 + Nut M10 ID.3

Washer M10 ID.4 + Nut M10 ID.3

Dural. roller ID.5 + Washer M10 ID.4 + Nut M10 ID.3

9

13,11,12

10

Origin.bolt

Original bolt. Rear place.