CF104

Honda X-ADV 750_2017-

| Kit for left side |  |  |  |  | Kit for right side |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | Description | Cat. No. | Pcs. |  | ID | Description | Cat. No. | Pcs. |
| 1 | Frame | CF104 | 1 |  | 1 | Frame | CF104 | 1 |
| 2 | Bolt M8x60 | 838 | 1 |  | 2 | Bolt M8x60 | 838 | 1 |
| 3 | Bolt M8x50 | 837 | 1 |  | 3 | Bolt M8x50 | 837 | 1 |
| 4 | Dural. roller d=31mm | 1264 | 1 |  | 4 | Dural. roller d=31mm | 1264 | 1 |
| 5 | Dural. roller d=19mm | 1265 | 1 |  | 5 | Dural. roller d=19mm | 1265 | 1 |
| 6 | Bolt M8x40 | 828 | 1 |  | 6 | Bolt M8x40 | 828 | 1 |
| 7 | Washer M8 flat | 800 | 2 |  | 7 | Washer M8 flat | 800 | 2 |
| 8 | Plastic cover | X5 | 3 |  | 8 | Plastic cover | X5 | 3 |
| 9 | RDmoto Sticker |  | 1 |  | 9 | RDmoto Sticker |  | 1 |

We recommend using medium strenght thread glue.

Mounting Instructions (same for both sides):

- Unscrew the original bolts from your motorcycle.

- Prepare the new frame all necesarry bolts.

- Attach the top part with M8 ID.2 bolt, then do the same with bottom part and M8 ID3 bolt + M8 ID.7 washer (insert the ID.4 roller between crash frame and motorcycle frame) and then mount the rear part with M8 ID.6 bolt + ID.7 washer (here use the ID.5 roller to put between crash and motorcycle frame).

- Proceed on the other side using the same procedure

- Finally, tighten all screws and attach ID.8 plastic covers.

ID6

ID3

ID2

Bolt M8 ID3 +  washer M8 ID7 + dural. roller ID 4 + ID8

Bolt M8 ID2 + ID8

Bolt M8 ID6 + Washer M8 ID7 + dural. roller ID5 + ID8

9

13,11,12

10

Origin.bolt

Original bolt. Rear place.