CF109

Honda CB 1000 R Neo Sport Café

| Kit for left side |  |  |  |  | Kit for right side |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | Description | Cat. No. | Pcs. |  | ID | Description | Cat. No. | Pcs. |
| 1 | AL insertion for protector | X122 | 1 |  | 1 | AL insertion for protector | X122 | 1 |
| 2 | Bolt M12x125x1.25 | 952 | 1 |  | 2 | Bolt M12x125x1.25 | 952 | 1 |
| 3 | Protector | PVPKN-0/16 | 1 |  | 3 | Protector | PVPKN-0/16 | 1 |
| 4 | Washer M10 flat | 804 | 1 |  | 4 | Washer M10 flat | 804 | 1 |
| 5 | Safety nut M10 | 810 | 1 |  | 5 | Safety nut M10 | 810 | 1 |
| 6 | Plastic cover M12 | X60 | 1 |  | 6 | Plastic cover M12 | X60 | 1 |
| 7 | RDmoto sticker |  | 1 |  | 7 | RDmoto sticker |  | 1 |
| 8 | Nut cap M10 | X35 | 1 |  | 8 | Nut cap M10 | X35 | 1 |
|  |  |  |  |  | 9 | Rod 400x10x30x30 |  | 1 |

We recommend using medium strenght thread glue during assembly.

Mounting instructions (same for both sides):

- Unscrew the original bolt from upper part of the frame.

- Prepare the protector with its bolt.

- Attach the upper part of ID.1 Frame using the ID.2 M12 bolt, but do not tighten yet.

- In the rear part, take the ID.8 Rod and push it through. Now attach the crash frame onto the rod with ID.4 washer and Id.5 M10 nut.

- Use the same instructions for installing the left side.

- Finally, tighten all bolts and fit the caps (ID.6).

ID2 + ID3 + ID6

9

13,11,12

10

ID9 + ID4 + ID5 + ID8

Origin.bolt

Original bolt. Rear place.