CF10

Suzuki DR650SE

| Mounting kit |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Note | Description | Cat.No | Quan. |  |  |  |  |  |  |  |
| 1 | Bolt M8x110 | 844 | 1 |  |  |  |  |  |  |  |
| 2 | Washer M8 | 802 | 4 |  |  |  |  |  |  |  |
| 3 | Bolt M8x45mm | 820 | 1 |  |  |  |  |  |  |  |
| 4 | Bolt M8x25mm | 831 | 2 |  |  |  |  |  |  |  |
| 5 | Washer M8 (spring) | 800 | 2 |  |  |  |  |  |  |  |
| 6 | aluminium. cylinder d=13mm | 130 | 1 |  |  |  |  |  |  |  |
| 7 | dural. cylinder d=60mm | 434 | 1 |  |  |  |  |  |  |  |
| 8 | dural. cylinder d=21mm | 1158 | 1 |  |  |  |  |  |  |  |
| 9 | Washer M10 | 805 | 1 |  |  |  |  |  |  |  |
| 10 | Stickers RDmoto |  | 2 |  |  |  |  |  |  |  |
| 11 | Adapter | X72 | 1 |  |  |  |  |  |  |  |

Mounting instruction:

During installation it is necessary to adjust the cutting off cooler holder..

Warning - the threads of the screws is recommended to use glue (blue color).

- Dismantle original bolt from frame.

- On the left side insert duralumin cylinder (note 7) and right side insert duralumin cylinder (note 8) and insert crash frames, this set fixed by original bolt.

- Remove bolt from coolant.

- Insert aluminum cylinder (note 6). Used bolt (no. 3 ) and washer (no. 9 and 2 ).

- Trought  the frame insert bolt (no. 1)  used washer (no. 2) and original nut.

- It is important to cut holder.

- Connect frames, used steel bar, bolts (no. 4) and washer (5 and 2).