CF11

Aprilia Pegaso 650

Fitting kit:

| ID | Description | Cat.No | Qty |
| --- | --- | --- | --- |
| 1 | Crash bars | CF11 | 2 |
| 2 | Steel adapter upper | X67 | 1 |
| 3 | Steel adapter lower | X66 | 1 |
| 4 | Dural.cylinder d=18mm | 1108 | 2 |
| 5 | Dural.cylinder d=13mm | 1109 | 2 |
| 6 | Bolt M10x70 | 851 | 2 |
| 7 | Bolt M6x40 | 865 | 2 |
| 8 | Bolt M8x25 | 831 | 2 |
| 9 | Washer spring M10 | 801 | 2 |
| 10 | Washer spring M6 | 985 | 2 |
| 11 | Washer spring M8 | 800 | 2 |
| 12 | Washer M10 | 804 | 2 |
| 13 | Nut M10 | 810 | 1 |
| 14 | Bolt M10x80 | 852 | 1 |
| 14 | Plastic cap | X4 | 2 |
| 15 | 3M tape | - | 2 |
| 16 | Stickers RDmoto | - | 2 |

Fitting instruction:

- Remove the original screw on the motorcycle frame under the cooler.

- Take a steel stirrup with welded pipe and by the screw together with the nut mount to the place under the cooler, this stirrup tighten only slightly.

- Now remove the original M6 screws on the engine, take the steel holder and insert screws together with washers into this holder.

- To this screws mount on the other hand duralumin cylinders length 13mm.

- These ready holder mount to the point where it joins the engine cover to the frame of the motorcycle , tighten only slightly.

- Remove the original M10 screws in the frame of the motorcycle (it's in the rear mounting).

- Take the crash frame for the left side , pass in the back of this frame screw M10x70mm together with a washer, from the rear of the crash frame put it on the screw duralumin cylinder length 18mm.

- In point of contact the crash frame with the motorcycle frame you can stick 3M tape.

- Insert the crash frame to the upper steel stirrup under the cooler and mount in the  rear of your motorcycle frame.

- Take the screw M8x25mm and connect the crash frame with the lower steel holder.

- Now should be easily attached crash frame on the left side.

- The same procedure mount crash frame  for the right side, then fix it.

Warning: We recommend use the glue of all threads. Medium strong.