CF16

Aprilia Pegaso 650´92-´95

Fitting kit:

| ID | Description | Cat.No | Qt; |
| --- | --- | --- | --- |
| 1 | Crash frames | - | 2 |
| 2 | Dural. spacer d=18mm | 1108 | 2 |
| 3 | Bolt M10x70 | 851 | 2 |
| 4 | Bolt M10x80 | 852 | 2 |
| 5 | Washer M10 | 804 | 6 |
| 6 | Washer spring M10 | 801 | 2 |
| 7 | Nut M10 | 810 | 2 |
| 8 | Plastic. cover | X4 | 2 |
| 9 | 3M Tape | - | 2 |
| 10 | RDmoto stickers | - | 2 |

Fitting instruction:

- Crash frames assembly the rear (side frame) with screws and washers ID.3, ID.5 and ID.6. Place the cylinder ID.2 under the frame. Place a spring washer under the bolt head first.

- Stick 3M tape on the place, where is frame touching with frame.

- For tighten the frames front the engine using bolts ID.4. Under the screw heads put pads. To tighten use the washers and nuts ID.5 ID.7

Caution: When installing, we recommend using glue on the threads (blue color).