CF67

Lower

Honda CRF 1000 Africa Twin DCT ´16-

| Fitting kit: ID | Description | Cat.N | Qt |
| --- | --- | --- | --- |
| 1 | Spacer l=5, hole dia. 8,5 | 433 | 1 |
| 2 | Spacer l=17, hole dia. 8,5 | 1190 | 1 |
| 3 | Bolt M8x130 | 884 | 1 |
| 4 | Washer M8 flat | 802 | 2 |
| 5 | Nut M8 safety | 809 | 1 |
| 6 | Silentblock | X34 | 1 |

Note: Use medium strong glue during assembly on the all threads.

Remove original bolt M8 from the frame of motorcycle.

Left side:

Put washer ID.4 on bolt. Now mount crash bar with spacer l=17mm on the motorcycle.

Right side:

Fit silentblock on the crash bar. Prepare spacer l=5mm, crash bar, washer and fit it on the right side using nut ID5.

Use original bolts for tightening frame.