CF79KD CF79R

Yamaha MT07

| Fitting kit for the left side |  |  |  |  | Fitting kit for the right side |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | Description | Cat.no. | Pcs |  | ID | Description | Cat.no. | Pcs |
| 1 | Crash frame |  | 1 |  | 1 | Crash frame |  | 1 |
| 2 | Screw M12x130x1.25 | 1027 | 1 |  | 2 | Screw M12x130x1.25 | 1027 | 1 |
| 3 | Slider | SL01 | 1 |  | 3 | Slider | SL01 | 1 |
| 4 | SL adaptér | 1137 | 1 |  | 4 | SL adapter | 1137 | 1 |
| 5 | Screw M6x25 | 877 | 3 |  | 5 | Screw M6x25 | 877 | 3 |
| 6 | Screw M4x16 | 876 | 2 |  | 6 | Screw M4x16 | 876 | 2 |
| 7 | Cover Black | CAPSL-K | 1 |  | 7 | Screw M10x40x1.25 | 941 | 1 |
| 8 | Dural. roller d=45mm | 1303 | 1 |  | 8 | Dural. roller d=45mm | 1303 | 1 |
|  |  |  |  |  | 9 | Cover black | CAPSL-K | 1 |
|  |  |  |  |  | 10 | Washer M10 flat | 804 | 1 |

We recommend to use glue on all threads (medium strong).

Instructions (for both sides):

- Remove original screw from upper part of frame.

- Prepare protector and screw.

- Attach the upper part with adapter and duralumin with roller (ID8), screw M12 (ID2)

- Remove original screw from lower part of frame.

- Attach lower part of frame using this original screw, but right screw M10x40x1.25(ID4)

- Tighten both screws.

- Attach the SL adapters using the M6 screws(ID.5) and then the cover, using screws M4(ID.6).

Original screws

left side,

Right side M10x40+Washer M10 flat

ID2

M12x130x1,25

ID8

9

13,11,12

10

9

13,11,12

10

Origin.bolt

Original bolt. Rear place.

Origin.bolt

Original bolt. Rear place.