CF93

KTM 690 Enduro R 2012-

| Kit for left side |  |  |  |  | Kit for right side |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Poz. | Popis | Kat.číslo | Počet |  |  | Poz. | Popis | Kat.číslo | Počet |  |
| 1 | Crash frame lower part | CF93 | 1 |  |  | 1 | Crash frame lower part | CF93 | 1 |  |
| 2 | Dural. cylinder d= 5mm | DV433 | 1 |  |  | 2 | Dural. cylinder d= 5mm | DV433 | 1 |  |
| 3 | Self-locking nut M6 | 948 | 3 |  |  | 3 | Self-locking nut M6 | 948 | 3 |  |
| 4 | Washer M6 – flat | 947 | 3 |  |  | 4 | Washer M6 – flat | 947 | 3 |  |
| 6 | Washer M10 – flat | 804 | 2 |  |  | 6 | Washer M10 – flat | 804 | 2 |  |
| 8 | Self-locking nut M10 | 810 | 2 |  |  | 8 | Bolt M10 x 170mm | 935 | 1 |  |
| 10 | Sticker RDmoto |  | 1 |  |  | 10 | Sticker RDmoto |  | 1 |  |
| 11 | Dural. cylinder d=24mm | 1248 | 1 |  |  | 11 | Dural. cylinder d=24mm | 1248 | 1 |  |
| 12 | Bolt M8x20 | 823 | 1 |  |  | 12 | Bolt M8x20 | 823 | 1 |  |
| 13 | Washer flat M8 | 802 | 1 |  |  | 13 | Washer flat M8 | 802 | 1 |  |
| 14 | Bolt M6x16 | 962 | 3 |  |  | 14 | Bolt M6x16 | 962 | 3 |  |
| 15 | Washer M6 | 987 | 3 |  |  | 15 | Washer M6 | 987 | 3 |  |
| 16 | Strut | X96 | 1 |  |  | 16 | Bolt M10 x180 | 978 | 1 |  |
| 17 | sheet metal cover | X107 | 1 |  |  |  |  |  |  |  |

Mounting instructions:

Install the crash frames as instructed on the attached photos.¨

Begin with installation of right side with the through-bolts. Left side is for washers and nuts.

Installation of the frames for both sides is the same.

Attention: We recommend using blue color medium strenght glue for threads of all bolts.

Bolt  M6   ID14 + Washer  M6  ID15,  +  Washer M6 flat ID4 + nut M6  ID.3 + sheet metal cover ID.17

Bolt M10x170mm ID8+ 2x Washer M10 (flat) ID6 + Self-locking nut ID.8

Strut ID16 + Bolt M10x180 ID16 + Dural. Cylinder ID11 + 2x Washer M10 ID6  + 1x Self-locking nut M10 ID.8

Bolt M8x20 ID12 + Washer M8 flat ID13 + Dural. cylinder ID.2

Bolt  M6   ID14 + Washer  M6  ID15,  +  Washer M6 flat ID4 + Nut M6  ID.3