CF97

Kawasaki Z900RS 2018-

| Kit for left side |  |  |  |  | Kit for right side |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | Description | Cat. no. | Pcs |  | ID | Description | Cat. no. | Pcs |
| 1 | Frame | CF97 | 1 |  | 1 | Frame | CF97 | 1 |
| 2 | Bolt M10x85x1.25 | 934 | 1 |  | 2 | Bolt M10x85x1.25 | 934 | 1 |
| 3 | Bolt M10x55x1.25 | 915 | 1 |  | 3 | Bolt M10x55x1.25 | 915 | 1 |
| 4 | Bolt M10x40x1.25 | 941 | 1 |  | 4 | Bolt M10x80x1.25 | 920 | 1 |
| 5 | Dural. roller d=43mm | 1246 | 1 |  | 5 | Dural. roller d=43mm | 1246 | 1 |
| 6 | Washer M10 flat | 804 | 2 |  | 6 | Washer M10 flat | 804 | 2 |
| 7 | Dural. roller d=30,5mm | 197 | 1 |  | 7 | Dural. roller d=30,5mm | 197 | 1 |
| 8 | Plastic plug | X19 | 1 |  | 8 | Plastic plug | X19 | 1 |

We recommend using medium strenght thread glue.

Instructions (identical for left and right side):

- Finish installation of one side completely before beginning with the other.

- Remove original bolt from the upper part of the frame.

- Screw the ID.2 bolt through crash frame ID.1 and ID.5 roller and snap it into the motorcycle frame (do not tighten).

- Remove the original bolt from rear and bottom part of the frame.

- Screw the left bottom part of frame using ID.4 bolt and right using ID.4 M10x60x1.25.

- Use ID.3 bolt and ID.7 roller to attach the rear top part.

- Finally, tighten all screws.

6

6

7

2

3

4

8

9

13,11,12

10

Origin.bolt

Original bolt. Rear place.