A11

Aprilia Caponord 1200

| ID. | Description | Cat.N | Qt |
| --- | --- | --- | --- |
| 1 | Steel nut M12 | X12/X2 | 1 |
| 2 | Alu spacer d=61mm | 31 | 1 |

Fitting kit left side:			             Fitting kit right side:

| ID. | Description | Cat.N | Qt |
| --- | --- | --- | --- |
| 1 | Steel nut M12 | X12/X2 | 1 |
| 2 | Alu spacer d=61mm | 31 | 1 |
|  | Steel connection bar M12x425x(1,25)x30x30 |  | 1 |

Assembly:

- Dismantle orignal bolt M12.

- Place steel bar into the frame M12x425x1,25mm

- Place alu. spacers ID.2 on this bar from booth sides.

- Tighten it by nuts ID.1.

Note: We recommend use medium strong glue for all threads.

1

3

2

1

4

3

2