A2SDM

Aprilia Tuono 2002-2005

| Kit for the left side |  |  |  |  |  |  |  | Kit for the right side |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | Description |  | Cat.no. | Pcs |  |  |  | ID | Description |  | Cat.no. | pcs |  |
| 1 | Bolt M10x55 |  | 1002 | 1 |  |  |  | 1 | Bolt M10x55 |  | 1002 | 1 |  |
| 2 | Dural. cylinder |  | DV11 | 1 |  |  |  | 2 | Dural. cylinder |  | DV11 | 1 |  |
| 3 | SLD adapter |  | 537B | 1 |  |  |  | 3 | SLD adapter |  | 537B | 1 |  |
| 4 | Bolt M6x20 |  | 1024 | 2 |  |  |  | 4 | Bolt M6x20 |  | 1024 | 2 |  |
| 5 | Podložka M6 |  | 1023 | 2 |  |  |  | 5 | Podložka M6 |  | 1023 | 2 |  |

Instructions:

- Remove original bolt from frame.

- Insert bolt M10 into SLD adapter and dural cylinder.

- Mount this kit into the frame of motorcycle

- Fit slider on the SLD adapter using two bolts M6 + washers.

We recommend to use the glue on all threads (medium strong).