A2S

Aprilia Tuono 2002-2005

| Kit for the left side |  |  |  |  |  |  |  | Kit for the right side |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | Description |  | Cat.no. | Pcs |  |  |  | ID | Description |  | Cat.no. | pcs |  |
| 1 | Bolt M10x25 |  | 868 | 1 |  |  |  | 1 | Bolt M10x25 |  | 868 | 1 |  |
| 2 | Slider adapter |  | 313 | 1 |  |  |  | 2 | Slider adapter |  | 313 | 1 |  |
| 3 | Bolt M6x25 |  | 877 | 3 |  |  |  | 3 | Bolt M6x25 |  | 877 | 3 |  |
| 4 | Bolt M4x16 |  | 876 | 2 |  |  |  | 4 | Bolt M4x16 |  | 876 | 2 |  |

Instructions:

- Remove original bolt from frame.

- Insert bolt M10 into adapter and mount it into the frame of motorcycle.

- Mount slider on the slider adapter using three bolts M6.

- Mount cover into the slider using two bolts M4.

We recommend to use the glue on all threads (medium strong).