A6

Aprilia RSV-4

Mounting kit Left Side:		     		Mounting kit Right Side:

| ID | Description | Cat.no |  | Pc |
| --- | --- | --- | --- | --- |
| 1 | Aluminium Insert M10 | X1 | 1 |  |
| 2 | Bolt M10x75mm | 1007 | 2 |  |
| 3 | Bolt M10x55mm | 1002 | 1 |  |
| 4 | Dural. roller d=20mm | 2 | 1 |  |
| 5 | Dural. roller d=20,7mm | 459 | 2 |  |
| 6 | Dural. adapter | 501 | 1 |  |
| 7 | Plastic cover M10 | X4 | 2 |  |

| ID | Description | Cat.No. | Pc |
| --- | --- | --- | --- |
| 1 | Aluminium Insert M10 | X1 | 1 |
| 2 | Bolt M10x90mm | 880 | 2 |
| 3 | Bolt M10x55mm | 1002 | 1 |
| 4 | Dural. roller d=20mm | 2 | 1 |
| 5 | Dural. roller d=20,7mm | 459 | 2 |
| 6 | Dural. Adapter | 500 | 1 |
| 7 | Plastic cover M10 | X4 | 2 |

1

1

2

2

2

2

3

3

4

4

5

5

5

5

6

6

7

7

7

7

Mounting Kit Left Side:

- Unmount the original side plastic and M10 bolts from motorcycle frame.

- Mount the dural. adapter (ID.6) onto the motorcycle frame (see place of fitment at the bottom picture).

- Now insert the dural. rollers (ID.5) under the dural. adapter and tighten the bolts (ID.2). Place ID.7 plastic covers on top of the bolts.

- Equip ID.3 bolt with ID.1 aluminium insert. Then insert ID.4 dural. roller underneath the protector and tighten with the ID.3 bolt.

Mounting Kit Right Side:

Mounting instructions for right side are the same as on the left.

- Unmount the original side plastic and M10 bolts from motorcycle frame.

- Mount the dural. adapter (ID.6) onto the motorcycle frame (see place of fitment at the bottom picture).

- Now insert the dural. rollers (ID.5) under the dural. adapter and tighten the bolts (ID.2). Place ID.7 plastic covers on top of the bolts.

- Equip ID.3 bolt with ID.1 aluminium insert. Then insert ID.4 dural. roller underneath the protector and tighten with the ID.3 bolt.

Note: We recommend using medium strenght thread glue during assembly.