A8

Aprilia RS125

Fitting kit left side:		     		       Fitting kit right side:

| ID | Description | Cat.no | QTY |
| --- | --- | --- | --- |
| 1 | Crash protektor | PH | 1 |
| 2 | Steel box | X2 | 1 |
| 3 | Bolt M12x40 | 857 | 1 |
| 4 | Dural.cylinder d=60 | 448 | 1 |
| 5 | Bolt M12x40 | 826 | 1 |
| 6 | Steel bracket | 694 | 1 |
| 7 | Bolts M8x45 | 836 | 2 |
| 8 | Washer M8 | 802 | 2 |
| 9 | AL cylinder d=38 | 133 | 1 |
| 10 | Nut M10 | 810 | 1 |
| 11 | Washer M10 | 804 | 1 |
| 12 | Stainless steel bar 10x295x30x30mm | - | 1 |

| ID | Description | Cat.no | QTY |
| --- | --- | --- | --- |
| 1 | Crash protektor | PH | 1 |
| 2 | Steel box | X2 | 1 |
| 3 | Bolt M12x40 | 857 | 1 |
| 4 | Dural.cylinder d=70 | 420 | 1 |
| 5 | Bolt M12x40 | 826 | 1 |
| 6 | Steel bracket | 695 | 1 |
| 7 | Bolts M8x45 | 836 | 2 |
| 8 | Washer M8 | 802 | 2 |
| 9 | AL cylinder d=38 | 133 | 1 |
| 10 | Nut M10 | 810 | 1 |
| 11 | Washer M10 | 804 | 1 |

Instruction to assembly-left side:

- Take off fairings and make a hole into the fairing.

- Dismantle original bolts M8 from frame and bolt M10 which is going through the engine.

- Take bolt M12x40 ID.5 and mount steel cylinder d=60 ID.4 from back side.

- Steel adapter ID.6 is mounting on the frame, use bolts and washer M8 (ID.7,ID.8).

- Between the steel adapter and frame insert AL cylinder d=38mm ID.9.

- Insert stainless steel bar ID.12 throught the engine and for gripping use nut M10 and washer M10 (ID.10,ID.11).

Instruction to assembly-right side:

- Take off fairings and make a hole into the fairing.

- Dismantle original bolts M8 from frame and bolt M10 which is going through the engine.

- Take bolt M12x40 ID.5 and mount steel cylinder d=70 ID.4 from back side.

- Steel adapter ID.6 is mounting on the frame, use bolts and washer M8 (ID.7,ID.8).

- Between the steel adapter and frame insert AL cylinder d=38mm ID.9. This cylinder put on the steel bar.

- Use nut and washer M10 (ID.10,ID.11) and fix stainless steel bar.

Instruction to assembly protectors.

- Now, put on the bike fairing with holes and moutning cash protectors.

- Insert steel box into the protektors and use bolts M12x40

Warning:

We recommend use the glue of all threads. Medium strong.