A9

Aprilia RS4 125

Fitting kit left side:		     		Fitting kit right side:

| ID | Description | Cat.N | Qty |
| --- | --- | --- | --- |
| 1 | Steel box M10 | X1 | 1 |
| 2 | Bolt M10x40 | 854 | 1 |
| 3 | Bolt M10x20 | 819 | 1 |
| 4 | Bolt M10x110 | 879 | 1 |
| 5 | Bar bracket | 698 | 1 |

| ID | Description | Cat.N | Qty |
| --- | --- | --- | --- |
| 1 | Steel box M10 | X1 | 1 |
| 2 | Bolt M10x40 | 854 | 1 |
| 3 | Nut M10 | 810 | 1 |
| 4 | Washer | 804 | 1 |
| 5 | Bar bracket | 699 | 1 |

4

3,4

2

1

2

5

1

5

3

Instruction to assembly:

- Take off left and right fairing.

- Dismantle original bolt M10 in front of engine.

- Dismantle original bolt from frame on the left side (red market on picture 1).

- Put bracket and fixed it by bolt ID.3 and insert through bolt ID.4

- On the right side dismantle orig.bolt.

- Put bracket for tighten use original bolt and nut wit washer ID.3 and ID.4

- Put fairing back to bike.

- Insert bolt ID.2 and steel box ID.1 into the protectors and tighten it on the bracket.

Warning: We recommend use the glue of all threads. Medium strong.

(picture1)