B10SDM

BMW S1000RR 2015-

| Kit for left side: |  |  |  |  | Kit for right side: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | Description | Cat.no | Qty |  |  | ID | Description | Cat.no | Qty |  |
| 2 | Dural. bracket | 537/B | 1 |  |  | 2 | Dural. bracket | 537/B | 1 |  |
| 3 | Bolts M6x20 | 1024 | 2 |  |  | 3 | Bolt M6x20 | 1024 | 2 |  |
| 4 | Dural.adapter | 1131 | 1 |  |  | 4 | Dural.adapter | 1132 | 1 |  |
| 5 | Bolt M10x120 | 881 | 1 |  |  | 5 | Bolt M10x85 | 981 | 1 |  |
| 6 | washer M10 - spring | 801 | 1 |  |  | 6 | washer M10 - spring | 801 | 1 |  |
| 7 | washer M6 | 1023 | 2 |  |  | 7 | washer M6 | 1023 | 2 |  |

Instruction to assembly-left side :

- Dismantle original bolt from frame.

- The washer ID.6 put on the botl ID.5.

- This bolt insert into the steel bracket ID.2 and dural.adapter ID.4

- This set mount on the frame.

- Use bolts ID.3 and washer ID.7 and fix the slider on the bracket.

Instruction to assembly-right side :

- Dismantle original bolt from frame.

- The washer ID.6 put on the botl ID.5.

- This bolt insert into the steel bracket ID.2 and dural.adapter ID.4

- This set mount on the frame.

- Use bolts ID.3 and washer ID.7 and fix the slider on the bracket.

Warning- the threads of the screws is recommended to use glue of medium strength (blue).