B18S

BMW S1000RR 2019-

| Kit for Left Side |  |  |  |  | Kit for Right Side |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | Description | Cat.No. | Pcs |  | ID |  | Description | Cat.No. | Pcs. |  |
| 1 | Slider adapter | 317 | 1 |  | 1 |  | Slider adapter | 317 | 1 |  |
| 2 | Screw M10x30mm | 882 | 1 |  | 2 |  | Screw M10x85mm | 981 | 1 |  |
| 3 | Safety nut M10 | 810 | 1 |  | 3 |  | Dural. sleeve d=34mm | 1333 | 1 |  |
| 4 | Dural. sleeve d=24mm | 1332 | 2 |  | 4 |  | Bolt M6x25 | 877 | 3 |  |
| 5 | Steel adapter | 757 | 1 |  | 5 |  | Bolt M4x12 | 876 | 2 |  |
| 6 | Screw M10x80mm | 853 | 2 |  |  |  |  |  |  |  |
| 7 | 3M Tape |  | 2 |  |  |  |  |  |  |  |
| 8 | Bolt M6x25 | 877 | 3 |  |  |  |  |  |  |  |
| 9 | Bolt M4x12 | 876 | 2 |  |  |  |  |  |  |  |

1

1

2

2

3

4

5

3

4

4

5

4

6

6

6

7

8

9

Right Side:

- Unscrew the original bolt as shown on attached picture.

- Bolt ID.2 insert into the  adapter ID.1 + ID.3. This set mounting the place on frame.

- Place slider and tighten it by bolts ID.4.

- Place alu. cover into the slider use bolts ID.5.

- Tighten using recommended tightening torque described in your motorcycle’s manual.

Left Side:

- Dismount the hood and original bolts as shown on pictures.

- Take the ID.5 steel adapter and insert ID.6 screws.

- From the rear adapter side, equip screws with ID.4 dural. sleeves.

- Mount this assembly onto the bike. Use the included 3M ID.7 tape when you feel its needed.

- Mount the hood back.

- Bolt ID.2 insert into the adapter ID.1.This set mounting the place on steel adapter

- Mount this assembly onto the steel adapter.

- From rear side of the adapter, use ID.3 nut to secure the assembly.

- Place slider and tighten it by bolts ID.8

- Place alu. cover into the slider use bolts ID.9.

- Tighten using recommended tightening torque described in your motorcycle’s manual.

Note – For threads of all screws we recommend using medium strenght thread glue (blue color).