B18

BMW S1000RR 2019-

| Kit for Left Side |  |  |  |  | Kit for Right Side |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | Description | Cat.No. | Pcs |  | ID |  | Description | Cat.No. | Pcs. |  |
| 1 | AL Insertion M10 | X1 | 1 |  | 1 |  | AL Insertion M10 | X1 | 1 |  |
| 2 | Screw M10x50mm | 849 | 1 |  | 2 |  | Screw M10x95mm | 822 | 1 |  |
| 3 | Dural. sleeve d=10mm | 1 | 1 |  | 3 |  | Dural. sleeve d=34mm | 1333 | 1 |  |
| 4 | Safety nut M10 | 810 | 1 |  |  |  |  |  |  |  |
| 5 | Dural. sleeve d=24mm | 1332 | 2 |  |  |  |  |  |  |  |
| 6 | Steel adapter | 757 | 1 |  |  |  |  |  |  |  |
| 7 | Screw M10x80mm | 853 | 2 |  |  |  |  |  |  |  |
| 8 | 3M Tape |  | 2 |  |  |  |  |  |  |  |

Right Side:

- Unscrew the original bolt as shown on attached picture.

- Take the protector and put in ID.1 Al insertion.

- Now insert ID.2 screw into this protector and from other side equip the screw with ID.3 dural. sleeve of 34mm lenght.

- Mount this assembly into motorcycle frame.

- Tighten using recommended tightening torque described in your motorcycle’s manual.

Left Side:

- Dismount the hood and original bolts as shown on pictures.

- Take the ID.6 steel adapter and insert ID.7 screws.

- From the rear adapter side, equip screws with ID.5 dural. sleeves.

- Mount this assembly onto the bike. Use the included 3M ID.8 tape when you feel its needed.

- Mount the hood back.

- Take the protector and put in the ID.1 Al insertion along with ID.2 screw.

- From rear side of the protector, put on ID.3 dural. sleeve.

- Mount this assembly onto the steel adapter.

- From rear side of the adapter, use ID.4 nut to secure the assembly.

Note – For threads of all screws we recommend using medium strenght thread glue (blue color).