B2SD

BMW K1200R/K1200R Sport

2005-

| LEFT SIDE KIT |  |  |  | RIGHT SIDE KIT |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | Description | Cat.no | Pcs |  | ID | Description | Cat.no | Pcs |  |  |
| 1 | SLD adapter | 537 | 1 |  | 1 | SLD adapter | 537 | 1 |  |  |
| 2 | Bolt M10x110x1,5mm | 879 | 1 |  | 2 | Bolt M10x100x1,5mm | 885 | 1 |  |  |
| 3 | Dural. cylinder 26mm | 418 | 1 |  | 3 | Dural. cylinder 16mm | 419 | 1 |  |  |
| 4 | Dural. cylinder 15mm | 9 | 1 |  | 4 | Dural. cylinder 15mm | 9 | 1 |  |  |
| 5 | Bolt M8x25 | 815 | 2 |  | 5 | Bolt M8x25 | 815 | 2 |  |  |
| 6 | Washer M8 | 800 | 2 |  | 6 | Washer M8 | 800 | 2 |  |  |
| 7 | Washer M10 | 801 | 1 |  | 7 | Washer M10 | 801 | 1 |  |  |

INSTRUCTIONS

- Put washer ID7, SLD adapter (ID1) and dural cylinder ID4+ID3 on the bolt ID2.

- Mount this kit into the frame of motorcycle.

- Fit diamond using two bolts ID5 and washers ID6.

We recommend to use safety glue on all threads.