B5SDM

BMW F800R 2009-

| ID | Description | Cat.N | Qt |
| --- | --- | --- | --- |
| 1 | Slider holder | 537/B | 2 |
| 2 | Alu. adapter | 11 | 2 |
| 3 | Alu. spacer | 452 | 2 |
| 4 | Bolt M10x130 | 885 | 2 |
| 5 | Bolt M6x20 | 1024 | 4 |
| 6 | Washer M6 | 1023 | 4 |

1

2

3

Assembly instruction:

- Assembly for left and right side is same.

4

Dismantle original bolts M10 from  chassi.

- Put together slider holder includ spacers and bolts ID.4. This kit assembly to the chassi.

- Fix sliders by bolts ID.5 + washers ID.6 from back side the slider holder.

5

6

Note: We recommend use glue of all threads during assembly. Medium strong.