B5

BMW F800R 2009-´11

| Kit for left side: |  |  |  |  | Kit for right side: |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | Description | Cat.N | Qt |  | ID | Description | Cat.N | Qt |  |
| 1 | Steel box M10 | X1 | 1 |  | 1 | Steel box M10 | X1 | 1 |  |
| 2 | Bolt M10x120mm | 881 | 1 |  | 2 | Bolt M10x120mm | 881 | 1 |  |
| 3 | Alu. spacer d=20mm | 2 | 1 |  | 3 | Alu. spacer d=26mm | 452 | 1 |  |

Assembly instruction:

- Dismantle original bolts M10 from chassi.

- Into the protectors insert steel sleeve ID.1. Under the protectors place alu. spacers ID.3. Shorter for left side.

- Tighten it by bolts ID.2.

Note: Use glue of all threads during assembly. Medium strong.