B6S

BMW S1000RR HP4

| ID. | Description | Cat.N | Qt |
| --- | --- | --- | --- |
| 1 | Steel bracket | 673 | 1 |
| 2 | Alu. adapter- shorter | 342 | 1 |
| 3 | Bolt M10x60 | 973 | 2 |
| 4 | Alu spacer 7,2mm | 1107 | 2 |
| 6 | 3M Tape | 3M | 1 |
| 7 | Bolt M10x40 | 854 | 1 |
| 9 | Bolt M6x25 | 877 | 3 |
| 10 | Bolt M4x16 | 876 | 2 |

| Kit for left side: Kit pro pravu stranu: ID. | Description | Cat.N | Qt |
| --- | --- | --- | --- |
| 1 | Steel bracket | 672 | 1 |
| 2 | Alu. adapter-longer | 343 | 1 |
| 3 | Bolt M10x65 | 943 | 3 |
| 4 | Washer M10 | 804 | 2 |
| 5 | Alu spacers 6,7mm | 1105 | 2 |
| 6 | 3M Tape | 3M | 1 |
| 9 | Bolt M6x25 | 877 | 3 |
| 10 | Bolt M4x16 | 876 | 2 |

Assembly left side:

Take off fairing from the bike.

- Dismantle original bolt from frame.

- Insert alu. spacers ID.5 into the frame place steel holder ID.1.

- For tighten use bolts M10x65 ID.3 and washers ID.4.

- U can stick 3M tape on the frame.

- Put fairing back the bike.

- Bolt ID.3 insert into the alu. special adapter ID.2.

- This kit is mounting on the steel adapter.

- Bolts M6x25 ID.9 is for mounting slider, place alu. cover into the slider use bolts M4x16 ID.10.

Assembly right side:

Take off fairing from the bike.

- Dismantle original bolt from frame.

- Insert alu. spacers ID.4 into the frame, place steel holder ID.1.

- For tighten use bolts M10x60 ID.3.

- U can stick 3M tape on the frame.

- Put fairing back the bike.

- Bolt ID.7 insert into the alu. special adapter ID.2.

- This kit is mounting on the steel adapter.

- Bolts M6x25 ID.9 is for mounting slider, place alu. cover into the slider use bolts M4x16 ID.10.

We recommend that you use glue to all threads- medium strength.