B7SDM

BMW S 1000R 2014-

Fitting kit left side:		     		Fitting kit right side:

| ID | Description | Cat.N | pcs |
| --- | --- | --- | --- |
| 1 | Steel Adapter | 692 | 1 |
| 2 | Dural. cylinder d=6,5 | 1105 | 1 |
| 3 | Bolt M10x60 | 850 | 1 |
| 4 | Washer M10 | 804 | 1 |
| 5 | Polyamid cylinder 20mm | X6 | 1 |
| 6 | Bolt M8x25 | 831 | 1 |
| 7 | Poly.spacer d=6mm | X64 | 1 |
| 8 | Bolt M6x16 | 962 | 1 |
| 9 | Washer M6 | 947 | 1 |
| 10 | Bolt M10x80 | 852 | 1 |
| 11 | Dural cylinder – 50mm | 13 | 1 |
| 12 | Washer stringed | 801 | 1 |
| 13 | 3M Tape | 3M | 2 |
| 14 | Washer M6 | 1023 | 2 |
| 15 | Bolt M6x20 | 1024 | 2 |
| 16 | Diamond adapter | 537/B | 1 |

| ID | Description | Cat.N | pcs |
| --- | --- | --- | --- |
| 1 | Steel adapter | 693 | 1 |
| 2 | Dural. cylinder d=7 | 1107 | 1 |
| 3 | Bolt M10x60 | 850 | 1 |
| 4 | Washer M10 | 804 | 1 |
| 5 | Polyamid.cylinder 20mm | X6 | 1 |
| 6 | Bolt M8x25 | 831 | 1 |
| 7 | Poly.spacer d=6mm | X64 | 1 |
| 8 | Bolt M6x16 | 962 | 1 |
| 9 | Washer M6 | 947 | 1 |
| 10 | Bolt M10x55 | 1002 | 1 |
| 11 | Dural. cylinder – 20mm | DV10 | 1 |
| 12 | Washer stringed | 801 | 1 |
| 13 | 3M Tape | 3M | 2 |
| 14 | Washer M6 | 1023 | 2 |
| 15 | Bolt M6x20 | 1024 | 2 |
| 16 | Diamond adapter | 537/B | 1 |

ID3+4+2

ID3+4+2

ID13

ID13

ID5+6

ID5+6

ID14+15

ID7+8+9

ID1

ID14+15

ID7+8+9

ID16

ID1

ID10+12

ID10+12

12

ID16

ID11

L

R

L

R

Instructions:

- We recommend to use the glue on all threads (blue color – medium srong).

- Remove fairing of motorcycle.

- Fit poly cylinder ID7 on the steel adapter ID1 using bolt ID8 and washer ID9.

- Fit poly cylinder ID5 on the back side of steel adapter ID1 using bolt ID6.

- Remove original bolt from frame of motorcycle.

- Stick 3M tape ID13 on the frame.

- Insert dural cylinder ID2* into the frame of motorcycle.

- Fit prepared steel adapter on the frame of motorcycle using bolt ID3 and washer ID4.

- Put fairing back on motorcycle.

- Prepare bolt ID10*.

  - Put washer ID12 on bolt.

  - Put diamond adapter ID16 on bolt.

  - Put dural cylinder ID11* on bolt.

- Now you can mount this kit into steel adapter.

- In the end fit protector using two bolts ID15 and washers ID14.

*(ID2 – longer cylinder is on the right side, ID10 – longer bolt is on the left side, ID11 – longer cylinder is on the left side)