B7S

BMW S 1000R 2014-

Fitting kit left side:		     		Fitting kit right side:

| ID | Description | Cat.N | Qt |
| --- | --- | --- | --- |
| 1 | Steel Adapter | 692 | 1 |
| 2 | Dural. spacer d=6,7 | 1105 | 1 |
| 3 | Bolt M10x60 | 850 | 1 |
| 4 | Washer M10 | 804 | 1 |
| 5 | Polyamid spacer 20mm | X6 | 1 |
| 6 | Bolt M8x25 | 831 | 1 |
| 7 | Poly.spacer d=6mm | X64 | 1 |
| 8 | Bolt M6x16 | 962 | 1 |
| 9 | Washer M6 | 947 | 1 |
| 10 | 3M Tape | 3M | 2 |
| 11 | Dural. adapter for SL01 | DV316 | 1 |
| 12 | Dural. spacer d=10 | DV1 | 1 |
| 13 | Bolt M10x40 | 854 | 1 |
| 14 | Bolt M6x25 | 877 | 3 |
| 15 | Bolt M4x16 | 876 | 2 |

| ID | Description | Cat.N | Qt |
| --- | --- | --- | --- |
| 1 | Steel adapter | 693 | 1 |
| 2 | Dural.spacer d=7,2 | 1107 | 1 |
| 3 | Bolt M10x60 | 850 | 1 |
| 4 | Washer M10 | 804 | 1 |
| 5 | Polyamid.spacer 20mm | X6 | 1 |
| 6 | Bolt M8x25 | 831 | 1 |
| 7 | Poly.spacer d=6mm | X64 | 1 |
| 8 | Bolt M6x16 | 962 | 1 |
| 9 | Washer M6 | 947 | 1 |
| 10 | 3M Tape | 3M | 2 |
| 11 | Dural.adapter for SL01 | DV292 | 1 |
| 12 | Bolt M10x35 | 847 | 1 |
| 13 | Bolt M6x25 | 877 | 3 |
| 14 | Bolt M4x16 | 876 | 2 |
| 15 |  |  |  |

7

11

10

12

7

1

9,8

3

10

1

11

3

10

11

6

4

13

5

2

2

15

14

4

5

6

7

8,9

13,14

12

12

Assembly instruction left side:

- Take left hand side fairing out from bike.

- Do assembly from back side poly spacer ID.7 On the adapter ID.1, use bolt ID.8. Under the bolt place washer ID.9.

- Do assembly from back side poly spacer ID.5 On the adapter ID.1, use bolt ID.6.

- Dismantle original bolt M10 from frame (as show on the picture).

- Stick 3M tape on the frame (as on the picture).

- Insert dural. spacer ID.2 into the frame.

- Place steel adapter ID1 with poly spacers on the bike. Use bolt ID.3 with washer ID.4 to assembly.

- Put fairing back to bike.

- Place bolt ID.13 into the dural. adapter ID.11 and spacer ID.12. This kit gripping on the steel adapter and tighten.

- Place slider and use bolts ID.14. Place cover and use bolts ID.15.

Assembly instruction right side:

- Take left hand side fairing out from bike.

- Do assembly from back side poly spacer ID.7 On the adapter ID.1, use bolt ID.8. Under the bolt place washer ID.9.

- Do assembly from back side poly spacer ID.5 On the adapter ID.1, use bolt ID.6.

- Dismantle original bolt M10 from frame (as show on the picture).

- Stick 3M tape on the frame (as on the picture).

- Insert dural. spacer ID.2 into the frame.

- Place steel adapter ID1 with poly spacers on the bike. Use bolt ID.3 with washer ID.4 to assembly.

- Put fairing back to the bike.

- Place bolt ID.12 into the dural. adapter ID.11. This kit gripping on the steel adapter and tighten.

- Place slider and use bolts ID.13. Place cover and use bolts ID.14.

We recommend to use glue of all threads, medium strong. (Blue colour Loctite 243)