B9-2SD

BMW F800R 2014-

| ID | Description | Cat.No | Pcs |
| --- | --- | --- | --- |
| 1 | SLD carrier | 537 | 2 |
| 2 | Dural. adapter | 11 | 2 |
| 3 | Spacer sleeve | 452 | 2 |
| 4 | Bolt M10x130 | 885 | 2 |
| 5 | Bolt M8x25 | 815 | 4 |
| 6 | Washer M8 | 800 | 4 |

Mounting instructions:

3

1

Mount according to the attached pictures.

- The procedure for the left and right sides is the same.

2

We recomment installing one side first and then installing the other side..

- Remove the original bolt M10 from the motorcycle frame.

- Insert the SLD carrier ID.1 on the bolt ID.4.

6

4

Slide the fitted adapter ID.2 under the carrier.

5

Insert the spacer sleeve ID. 3 into the frame.

- Mount the protector to the frame.

- Insert the bolts ID.5 including the washers ID.6 from the rear and mount the slider.

Warning: We recommend using medium strength (blue) thread glue during assembly.