B9-2S

BMW F800R 2014-

Kit for the left side:				Kit for the right side:

| ID. | Description | Cat.no. | pcs |
| --- | --- | --- | --- |
| 1 | Adapter moto | DV-316 | 1 |
| 2 | Bolt M6x25 | 877 | 3 |
| 3 | Bolt M4x16 | 876 | 2 |
| 4 | Bolt M10x90mm | 880 | 1 |
| 5 | Washer M10- perova | 801 | 1 |

| ID. | Description | Cat.no. | pcs |
| --- | --- | --- | --- |
| 1 | Adapter moto | DV-316 | 1 |
| 2 | Bolt M6x25 | 877 | 3 |
| 3 | Bolt M4x16 | 876 | 2 |
| 4 | Bolt M10x90mm | 880 | 1 |
| 5 | Washer M10 - pérová | 801 | 1 |
| 6 | Dural spacer d=6mm | 453 | 1 |

Installation – left side:

- Remove original bolt from the frame of motorcycle.

- Insert bolt+washer (ID4+ID5) into dural adapter (ID1)

- Mount this kit on the frame of motorcycle.

- Fit slider using bolts (ID2). Fit coloured cover on the slider using bolts (ID3)

Installation – right side:

- Remove original bolt from the frame of motorcycle.

- Insert bolt+washer (ID4+ID5) into dural adapter (ID1)

- Insert dural spacer (ID6) into the frame.

- Mount this kit on the frame of motorcycle.

- Fit slider using bolts (ID2). Fit coloured cover on the slider using bolts (ID3)

We recommend to use the glue on all threads (medium strong – blue colour).