B9

BMW F800 R 2014-

Fitting kit left side:		     		Fitting kit right side:

| ID | Description | Cat.N | Qt |
| --- | --- | --- | --- |
| 1 | Steel adapter | 675 | 1 |
| 2 | Alu.spacer d=29,5 | 1154 | 1 |
| 3 | Alu. spacer d=6mm | 1105 | 1 |
| 4 | Bolt M10x110 | 879 | 1 |
| 5 | Bolt M10x85 | 981 | 1 |
| 6 | Washer M10 | 804 | 2 |
| 7 | Plastic cap | X4 | 2 |
| 8 | Bolt M10x35 | 847 | 1 |
| 9 | Steel box | X1 | 1 |

| ID | Description | Cat.N | Qt |
| --- | --- | --- | --- |
| 1 | Steel adapter | 675 | 1 |
| 2 | Alu. spacer d=41mm | 1153 | 1 |
| 3 | Alu. spacer d=13mm | 1155 | 1 |
| 4 | Bolt M10x120 | 881 | 1 |
| 5 | Bolt M10x90 | 880 | 1 |
| 6 | Washer M10 | 804 | 2 |
| 7 | Plastic cap | X4 | 2 |
| 8 | Bolt M10x35 | 847 | 1 |
| 9 | Steel box | X1 | 1 |

Assembly left side:

- Dismantle original bolts M10 from chassi.

- Insert bolt ID.4 front gripping with washer ID.6 into the steel adapter ID.1. Rear gripping use bolt ID.5 + washer ID6.

- Into the frame place spacers. Front spacer is ID.2 rear is ID.3.

- Tighten bolts M10.

- Place plastic covers into head bolts.

- Into teh crash bobbin insert steel box ID.9 and fix protector by bolt ID.8

Assembly right side:

- Dismantle original bolts M10 from chassi.

- Insert bolt ID.4 front gripping with washer ID.6 into the steel adapter ID.1. Rear gripping use bolt ID.5 + washer ID6.

- Into the frame place spacers. Front spacer is ID.2 rear is ID.3.

- Tighten bolts M10.

- Place plastic covers into head bolts.

- Into teh crash bobbin insert steel box ID.9 and fix protector by bolt ID.8

During assembly use medium strong glue for all treads.