BN3-2

Benelli BN600R 2013-

Mounting Kit Left Side:		     		Mounting Kit Right Side:

| ID | Popis | Kat.č. | Ks |
| --- | --- | --- | --- |
| 1 | Steel Insert M10 | X1 | 1 |
| 2 | Bolt M10x160 (155mm) | 979 | 1 |
| 3 | Dural. roller | DV5 | 1 |
| 4 | Washer M10 | 804 | 1 |
| 5 | Nut M10 | 810 | 1 |

| ID | Popis | Kat.č. | Ks |
| --- | --- | --- | --- |
| 1 | Steel Insert M10 | X1 | 1 |
| 2 | Bolt M10x160 (155mm) | 979 | 1 |
| 3 | Dural. roller | DV5 | 1 |
| 4 | Washer M10 | 804 | 1 |
| 5 | Nut M10 | 810 | 1 |

Mounting Instructions Left Side:

- Dismount original M10 bolt from motorcycle frame.

- Equip ID. 2 bolt with ID.1 steel insert.

- Take the equipped bolt and push it into the protector, underneath which you put ID.3 roller.

- Push this assembly through the engine and frame and tighten from back side, using ID.5 nut and ID.4 washer.

Mounting Instructions Right Side:

- Dismount original M10 bolt from motorcycle frame.

- Equip ID. 2 bolt with ID.1 steel insert.

- Take the equipped bolt and push it into the protector, underneath which you put ID.3 roller.

- Push this assembly through the engine and frame and tighten from back side, using ID.5 nut and ID.4 washer.

Note: We recommend using medium strenght thread glue during assembly (blue color).