BN3SDM

Benelli BN600R 2013-

Fitting kit right side:			     		Fitting kit left side:

| ID | Description | Cat.N | QTY |
| --- | --- | --- | --- |
| 1 | Bolt M10x110 | 879 | 1 |
| 2 | Washer M10 spring | 801 | 1 |
| 3 | Dural. adapter | 149 | 1 |
| 4 | Bracket for SLDM | 537/B | 1 |
| 5 | Bolt M6x20 | 1024 | 2 |
| 6 | Washer M6 | 1023 | 2 |
| 7 | Dural. cylinder 20mm | DV2 | 1 |

| ID | Description | Cat.N | QTY |
| --- | --- | --- | --- |
| 1 | Bolt M10x85 | 981 | 1 |
| 2 | Washer M10 spring | 801 | 1 |
| 3 | Dural. adapter | 149 | 1 |
| 4 | Bracket for SLDM | 537/B | 1 |
| 5 | Bolt M6x20 | 1024 | 2 |
| 6 | Washer M6 | 1023 | 2 |

Fitting kit left side:

- Dismantle original bolt from frame.

- Take bolt ID.1. Insert washer ID.2 and bracket ID.4 on the bolt.

- Insert dural. adapter ID.3 and under the adapter put dural.cylinder. ID.7.

- This set, mount into the frame. Original dural. cylinder leave in frame.

- Use bolts ID.5 and washers ID.6 for gripping the slider.

Fitting kit right side:

- Dismantle original bolt from frame.

- Take bolt ID.1. Insert washer ID.2 and bracket ID.4 on the bolt.

- Insert dural. adapter ID.3.

- This set, mount into the frame. Original dural. cylinder leave in frame.

- Use bolts ID.5 and washers ID.6 for gripping the slider.

Warning: Use glue of all threads. Medium strong.