BN3

Benelli BN600R 2013-

Fitting kit left side:		     		Fitting kit right side:

| ID | Description | Cat.No | QTY |
| --- | --- | --- | --- |
| 1 | Steel box M10 | X1 | 1 |
| 2 | Bolt M10x110 | 879 | 1 |
| 3 | Dural.cylinder 20mm | DV2 | 1 |

| ID | Description | Cat.N | QTY |
| --- | --- | --- | --- |
| 1 | Steel box M10 | X1 | 1 |
| 2 | Bolt M10x85 | 981 | 1 |

Instruction to assembly:

- Take out original bolt M10, from frame.

- Orig. dural cylinder leave in frame.

- Take bolt ID.2, and put on it steel box. ID.1.

- Into the protector insert bolt with steel box, under the protector insert dural. cylinder ID.3.

- This set fitting on frame.

Instruction to right side:

- Take out original bolt M10, from frame.

- Orig. dural cylinder leave in frame.

- Take bolt ID.2, and put on it steel box. ID.1.

- Into the protector insert bolt with steel box.

- This set fitting on frame.

Warning: We recommend use the glue of all threads. Medium strong.