BN5SDM

Benelli BN302  ´15-

| Left side kit: |  |  |  |  | Right side kit: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID. | Description | Cat.no. | Pcs |  |  | ID. | Description | Cat.no. | Pcs |  |
| 1 | Dural. cylinder into the frame | 1182 | 1 |  |  | 1 | Dural. cylinder into the frame | 1182 | 1 |  |
| 2 | Dural. cylinder for slider | DV9 | 1 |  |  | 2 | Dural. cylinder for slider | DV9 | 1 |  |
| 3 | Bolt M10x85 | 981 | 1 |  |  | 3 | Bolt M10x85 | 981 | 1 |  |
| 4 | SLDM adapter | 537/B | 1 |  |  | 4 | SLDM adapter | 537/B | 1 |  |
| 5 | Bolt M6x20 | 1024 | 2 |  |  | 5 | Bolt M6x20 | 1024 | 2 |  |
| 6 | Washer M6 | 1023 | 2 |  |  | 6 | Washer M6 | 1023 | 2 |  |

Mounting kit:

- Remove original bolt from the frame.

- Insert dural cylinder ID1 into the frame.

- Prepare bolt M10 and put dural cylinder for slider ID2 and SLD adapter ID4 on it.

- Mont this kit into the frame of motorcycle.

- Fit slider using two bolts ID5 and washers ID6.

1

2

3

4

5

3

1

2

6

6

5

4

Během montáže doporučujeme použít lepidlo na závity (středně silné).