BN8SD

Benelli BN 125 2019-

Mounting kit – Left Side:		     		Mounting kit – Right Side

| ID | Description | Cat.No | Pcs |
| --- | --- | --- | --- |
| 1 | Lock nut M10 | 810 | 1 |
| 2 | Dural. sleeve l=20mm | 1308 | 1 |
| 3 | Dural. SLD adapter | 537 | 1 |
| 4 | Bolt M8x25 | 815 | 2 |
| 5 | Washer M8 | 876 | 2 |
| 6 | Dural. sleeve l=30mm | DV11 | 1 |
| 7 | Rod 10x375x1,25 |  | 1 |

| ID | Description | Cat.No | Pcs |
| --- | --- | --- | --- |
| 1 | Lock nut M10 | 810 | 1 |
| 2 | Dural. sleeve l=20mm | 1308 | 1 |
| 3 | Dural. SLD adapter | 537 | 1 |
| 4 | Bolt M8x25 | 815 | 2 |
| 5 | Washer M8 | 876 | 2 |
| 6 | Dural. sleeve d=30mm | DV11 | 1 |

1

1

2

2

3

3

7

4

4

4

5

6

6

Mounting instructions (both sides):

- Remove original bolt M10 from the motorcycle frame.

- Insert the M10 (ID7) rod through the frame.

- Place dural. sleeves (ID2 + ID 6) and SLD adapter (ID3) on both sides of the rod. Fasten the assembly with the nut M10 ID.1 (see photo).

- Finally, fasten the slider using M8 bolt (ID4) and M8 washers (ID5).

Warning - We recommend using medium strength thread glue during assembly.