BN8

Benelli BN 125 2019-

Mounting Kit Left Side:		     		Mounting Kit Right Side:

| ID | Description | Cat. No. | Pcs |
| --- | --- | --- | --- |
| 1 | Steel Insert M10 | X1 | 1 |
| 2 | Dural. Roller d=20mm | 1308 | 1 |
| 3 | Steel Nut M10 | X11 | 1 |
| 4 | Rod 10x400x1,25 |  | 1 |

| ID | Description | Cat.No. | Pcs |
| --- | --- | --- | --- |
| 1 | Steel Insert M10 | X1 | 1 |
| 2 | Dural. Roller d=20mm | 1308 | 1 |
| 3 | Steel Nut M10 | X11 | 1 |

1

1

2

2

3

3

4

Mounting Instructions:

- Dismount original M10 rod from motorcycle frame.

- Take the ID.4 M10 rod and push it through the frame.

- Put on ID.2 roller from both sides of the rod.

- Push the ID.1 steel insert into protector and mount this assembly onto the rod from both sides.

- As a last step, attach the full set using M10 ID.3 steel nut.

Note: We recommend using medium strenght thread glue during assembly (blue color).