BN9S

Benelli 752S 2019-

| Kit for the Left Side |  |  |  |  | Kit for the Right Side |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | Description | Cat.no. | Pcs |  |  | ID | Description | Cat.no. | Pcs |  |
| 1 | Bolt M10x160mm | 979 | 1 |  |  | 1 | Bolt M10x30mm | 882 | 1 |  |
| 2 | Dural. sleeve length 40,5mm | 1327 | 1 |  |  | 2 | Dural. bolt length 20mm | 447 | 1 |  |
| 3 | Safety Nut M10 | 810 | 1 |  |  | 3 | Dural. Sleeve length 39mm | 1328 | 1 |  |
| 4 | Bolt M6x25mm | 877 | 3 |  |  | 4 | Bolt M6x25mm | 877 | 3 |  |
| 5 | Bolt M4x12mm | 876 | 2 |  |  | 5 | Bolt M4x12mm | 876 | 2 |  |
| 6 | Dural. adapter | 317 | 1 |  |  | 6 | Dural. adapter | 317 | 1 |  |

Mounting Kit:

Mounting of the Right Side:

- Remove the original plastic cover in the motorcycle frame

- Mount the Dural. sleeve (ID. 3) into the frame

- Install the Dural. bolt (ID. 2) into the Dural. sleeve

- Insert the bolt into the Dural. adapter (ID 6) and mount it in the thread in the Dural. bolt

- Put on a slider and fasten the bolts (ID.4)

- Insert the cover into the slider and fasten the bolts (ID 5)

Note – we recommend using medium strength (blue) safety glue on the threads of all screws.

Mounting of the Left Side:

- Remove the original plastic cover in the motorcycle frame

- Insert Dural. sleeve (ID 2) into the frame

- Insert the Bolt (ID 1) into the dural. adapter (ID 6) and pass it through the motorcycle frame. Secure with the nut (ID 3) from the back.

- Put on a slider and fasten the bolts (ID.4)

- Insert the cover into the slider and fasten the bolts (ID 5)

Note – we recommend using medium strength (blue) safety glue on the threads of all screws.