BN9

Benelli 752S 2019-

| Kit for the Left Side |  |  |  |  | Kit for the Right Side |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | Description | Cat.no. | Pcs |  |  | ID | Description | Cat.no. | Pcs |  |
| 1 | AL insert M10 | X1 | 1 |  |  | 1 | AL insert M10 | X1 | 1 |  |
| 2 | Bolt M10x170mm | 935 | 1 |  |  | 2 | Bolt M10x40mm | 854 | 1 |  |
| 3 | Dural. sleeve length 40,5mm | 1327 | 1 |  |  | 3 | Dural. Bolt length 20mm | 447 | 1 |  |
| 4 | Safety Nut M10 | 810 | 1 |  |  | 4 | Dural. Sleeve length 39mm | 1328 | 1 |  |

Mounting Kit:

Mounting of the Right Side:

- Remove the original plastic cover in the motorcycle frame

- Mount the Dural. sleeve (ID. 4) into the frame

- Install the Dural. bolt (ID. 3) into the Dural. sleeve

- Insert the AL insert (ID. 1) into the crash protector

- Insert the bolt (ID. 2) through the crash protector and mount it to the Dural. bolt on the motorcycle

Note – we recommend using medium strength (blue) safety glue on the threads of all screws.

Mounting of the Left Side:

- Remove the original plastic cover in the motorcycle frame

- Insert Dural. sleeve (ID 3) into the frame

- Insert AL insert (ID 1) in the crash protector

- Insert the bolt (ID 2) through the crash protector and mount it to the motorcycle frame, secure with a nut from the back

Note – we recommend using medium strength (blue) safety glue on the threads of all screws.