C3

Cagiva Raptor 650 ´01-

Kit for left side:

| ID | Description | Cat.no | Qty |
| --- | --- | --- | --- |
| 2 | Steel box | X2 | 1 |
| 3 | Bolt M12x130x1,25 | 1027 | 1 |
| 4 | Dural.cylinder d=85mm | 1192 | 1 |

Kit for right side:

| ID | Description | Cat.no | Qty |
| --- | --- | --- | --- |
| 2 | Steel box | X2 | 1 |
| 3 | Bolt M12x150x1,25 | 984 | 1 |
| 4 | Dural.cylinder d=105mm | 1193 | 1 |

Instruction to assembly:

- Take out original bolt from frame on the left side.

- Into the frame insert dural.cylinder ID.4 (short).

- Insert steel box into the bobbins ID.2. Use the bolt ID.3 and mount this set on the bike.

- Instruction for right side is same.

We recommend use the glue of all threads (medium strong).

1

4

3

1

2

3

4