D1(10)

Ducati Monster 600/750/900 (-2000)

| ID. | Description | Cat.no | pcs |
| --- | --- | --- | --- |
| 1 | Connecting rod M10x(1,25)x395x30x30 |  | 1 |
| 2 | Steel insert | X1 | 2 |
| 3 | Special nut M10x1,25 | X11 | 2 |
| 4 | Dural roller d=46mm | 25 | 1 |
| 5 | Dural roller d=48mm | 26 | 1 |

- Remove orignial screws.

- Prepare set same as on the picture below.

  - Insert steel (ID1) rod into frame.

  - On both sides put dural rollers(ID4 and ID5) on the rod.

  - Don’t forget to place steel insert into protector.

  - Tighten it with special nut (ID3).

front

3

2

3

5

4

2

1