D10SDM

Ducati Hyperstrada 821 (13-)

| Mounting Kit for Left Side |  |  |  |  |  |  |  |  | Mounting Kit for Right Side |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | Description |  | Cat. No. |  | Pcs. |  |  |  | ID | Description |  | Cat. No. |  | Pcs. |  |  |
| 1 | Slider adapter |  | 537/B |  | 1 |  |  |  | 1 | Slider adapter |  | 537/B |  | 1 |  |  |
| 2 | Safety Nut M12 |  | 811 |  | 1 |  |  |  | 2 | Safety Nut M12 |  | 811 |  | 1 |  |  |
| 3 | Dural. roller – 85mm |  | 1199 |  | 1 |  |  |  | 3 | Dural. roller – 85mm |  | 1199 |  | 1 |  |  |
| 4 | Dural. roller d = 30mm |  | 171 |  | 1 |  |  |  | 4 | Dural. roller d = 30mm |  | 171 |  | 1 |  |  |
| 5 | Bolt M6x20 |  | 1024 |  | 2 |  |  |  | 5 | Bolt M6x20 |  | 1024 |  | 2 |  |  |
| 6 | Washer M6 |  | 1023 |  | 2 |  |  |  | 6 | Washer M6 |  | 1023 |  | 2 |  |  |
| 7 | Steel Rod 12x495x30x30 |  | - |  | 1 |  |  |  | 7 |  |  |  |  |  |  |  |

Instructions:

- Unscrew original bolts from the frame.

- Push the ID.7 Steel rod through the frame.

- Equip the ID.3 and ID.4 rollers onto both ends of the rod.

- Fit the ID.1 slider adapters after the dural. rollers and tighten the whole assembly with ID.2 M12 nuts.

- Finally, mount the slider onto the slider adapter with two ID.5 M8 bolts and ID.6 washers.

Use medium strenght blue color thread glue during installation.