D12S

Ducati S4R/S4RS/S4

| Kit for the left side |  |  |  |  |  |  |  |  | Kit for the right side |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | Description |  | Cat.no. |  | Pcs |  |  |  | ID | Description |  | Cat.no. |  | pcs |  |  |
| 1 | Slider adapter |  | DV314 |  | 1 |  |  |  | 1 | Slider adapter |  | DV314 |  | 1 |  |  |
| 2 | Steel nut M12 |  | X12 |  | 1 |  |  |  | 2 | Steel nut M12 |  | X12 |  | 1 |  |  |
| 3 | Dural. cylinder – 86mm |  | 1161 |  | 1 |  |  |  | 3 | Dural. cylinder – 66mm |  | 1162 |  | 1 |  |  |
| 4 | Bolt M6x25 |  | 877 |  | 3 |  |  |  | 4 | Bolt M6x25 |  | 877 |  | 3 |  |  |
| 5 | Bolt M4x16 |  | 876 |  | 2 |  |  |  | 5 | Bolt M4x16 |  | 876 |  | 2 |  |  |
| 6 | Steel rod 12(1,25)x435x30x30 |  | - |  | 1 |  |  |  | - | - |  | - |  | - |  |  |

Instructions:

- Remove original bolt from frame of motorcycle.

- Insert steel rod (ID6) trough the frame.

- Mount cylinders (ID3 – longer on the left side) on both ends of steel rod.

- Mount slider adapters (ID1) behind cylinders and thighten this set using steel nut (ID2).

- Fit slider on the slider adapter using three bolts M6 (ID4).

- Fit coloured cover using two M4 bolts (ID5).

ID3

ID1

ID6

ID2