D13S

Ducati Scrambler 2015-

| Left side kit |  |  |  |  | Right side kit |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | Description | Cat.no. | pcs |  |  | ID | Description | Cat.no. | pcs |  |
| 1 | Nut M12 | X12 | 1 |  |  | 1 | Nut M12 | X12 | 1 |  |
| 2 | Dural. cylinder - 68,5mm | 1212 | 1 |  |  | 2 | Dural. cylinder - 78,5mm | 1213 | 1 |  |
| 3 | Dural. adapter - 48mm | 314 | 1 |  |  | 3 | Dural. adapter - 48 | 314 | 1 |  |
| 4 | Bolt M6x25 | 877 | 3 |  |  | 4 | Bolt M6x25 | 877 | 3 |  |
| 5 | Bolt M4x16 | 876 | 2 |  |  | 5 | Bolt M4x16 | 876 | 2 |  |
| 6 | Steel rod 12(1,25)x405x30x30 |  | 1 |  |  |  |  |  |  |  |

Instructions:

- Unsrew original bolt M12 from the frame of motorcycle.

- Put steel rod M12 through the frame.

- Put dural cylinders (ID2) and adapters (ID3) on both ends of steel rod.

- Tighten both ends using nut M12.

- Mount slider on the adapter using three bolts M6.

- Mount coloured cover into the slider using two bolts M4.

We recommend to use the glue on all threads (medium strong).