D15

Ducati Hyperstrada 939/Hypermotard  821/939  2016-

Left side mounting kit:		     	Right side mounting kit:

| ID | Description | Cat.no | pcs |
| --- | --- | --- | --- |
| 1 | Steel nut M12 | X12/X2 | 1 |
| 2 | Dural. Cylinder–90mm | 1200 | 1 |
|  | Stainless steel rod M12(1,25)x435x30x30xmm |  | 1 |

| ID | Description | Cat.no. | pcs |
| --- | --- | --- | --- |
| 1 | Steel nut M12 | X12/X2 | 1 |
| 2 | Dural. Cylinder–85mm | 1199 | 1 |

Instructions:

- Remove original bolt M12.

- Push stainless steel rod M12x435x1,25mm trough the frame of motorcycle.

- Put dural. Cylinders (ID2) on both ends of stainless steel rod. Longer cylinder is on the left side.

- protectors and put this kit on both ends of stainless steel rod.

- Tighten both protectors using steel nut M12 (ID1).

1

1

2

2

We recommend to use the glue on all threads (blue colour – medium strong).