D16S

Ducati X-Diavel

| Kit for the left side |  |  |  |  | Kit for the right side |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | Description | Cat.no. | Pcs. |  |  | ID | Description | Cat.no. | Pcs. |  |
| 1 | Slider adapter | 315 | 1 |  |  | 1 | Slider adapter | 315 | 1 |  |
| 2 | Dural. threaded cylinder 65,7mm | 1214 | 1 |  |  | 2 | Dural. threaded cylinder 68mm | 1215 | 1 |  |
| 3 | Dural bolt M22x1,5mm | 447 | 1 |  |  | 3 | Dural bolt M22x1,5mm | 447 | 1 |  |
| 4 | Bolt M6x25 mm | 877 | 1 |  |  | 4 | Bolt M6x25 mm | 877 | 1 |  |
| 5 | Bolt M4x16 mm | 876 | 1 |  |  | 5 | Bolt M4x16 mm | 876 | 1 |  |
| 6 | Bolt M10x45mm | 848 | 1 |  |  | 5 | Bolt M10x45mm | 848 | 1 |  |
| 7 | Bolt M12x75x1,25mm | 914 | 1 |  |  | 6 | Bolt M12x75x1,25mm | 914 | 1 |  |

Instructions:

- We recommend to use the glue on all threads.

- Unscrew original bolt from the frame of motorcycle.

- Prepare kit according to the picture below.

Note: Longer threaded cylinder is on the right side.

- Mount this kit into the frame of motorcycle.

- Mount slider using three bolts M6.

- In the end fit coloured cover using two bolts M4.

ID7

ID6