D17SD

Ducati Monster 797 2017-

| Mounting Kit - Left Side: |  |  |  |  | Mounting Kit - Right Side: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | Description | Cat.No | Pcs |  |  | ID | Description | Cat.No | Pcs |  |
| 1 | SLD adapter | 537 | 1 |  |  | 1 | SLD adapter | 537 | 1 |  |
| 2 | Lock nut M12 | 811 | 1 |  |  | 2 | Lock nut M12 | 811 | 1 |  |
| 3 | Dural. sleeve l=81,5mm | 1242 | 1 |  |  | 3 | Dural. sleeve =81,5mm | 1242 | 1 |  |
| 4 | Bolt M8x25 | 815 | 2 |  |  | 4 | Bolt M8x25 | 815 | 2 |  |
| 5 | Washer M8 | 800 | 2 |  |  | 5 | Washer M8 | 800 | 2 |  |
| 6 | Dural. sleeve l=30mm | 171 | 1 |  |  | 6 | Dural. sleeve l=30mm | 171 | 1 |  |
| 7 | Stainless steel rod 12x490x30x30 |  | 1 |  |  |  |  |  |  |  |

Mounting instructions:

- Remove original bolt M12 from the frame.

- Insert the rod M12x490x30mm ID.7 through the frame

- Place the dural. sleeves ID.3 + ID.6 on the both sides of the rod.

- Install the SLD adapter ID.1 and tighten the assembly with the nut M12 ID.2.

- Insert the slider and fasten it using bolts M8 ID.4 and washers ID.5.

Warning: We recommend using medium strength (blue) thread glue during assembly.