D17

Ducati Monster 797 2017-

| Left side mounting kit: |  |  |  |  | Right side mounting kit: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | Description | Cat. No | pcs |  |  | ID | Description | Kat.no | pcs |  |
| 1 | Steel nut M12 | X12/X2 | 1 |  |  | 1 | Steel nut M12 | X12/X2 | 1 |  |
| 2 | Dural. cylinder d=81,5mm | 1242 | 1 |  |  | 2 | Dura. Cylinder d=81,5mm | 1242 | 1 |  |
| 3 | Stainless steel rod 12(1,25)x425x30x30 |  | 1 |  |  |  |  |  |  |  |

- Remove origibal bolt M12

- Push stainless steel rod M12x425x1,25mm trough the frame of motorcycle.

- Put dural. Cylinders (ID2) on both ends of stainless steel rod.

- protectors and put this kit on both ends of stainless steel rod

- Tighten both protectors using steel nut M12 (ID1).

We recommend to use the glue on all threads (blue colour – medium strong).