D20SD

Ducati Multistrada 1260S 2018-

Mounting Kit – Left Side:		     		Mounting Kit – Right Side:

| ID | Description | Cat.No | Pcs |
| --- | --- | --- | --- |
| 1 | Galvanized steel nut | X117 | 1 |
| 2 | Bolt M10x110x1,25 | 928 | 1 |
| 3 | Dural. sleeve l=22mm | 1300 | 1 |
| 4 | Dural. sleeve l=4mm | 1260 | 1 |
| 5 | Dural. sleeve l=70mm | 15 | 1 |
| 6 | Bolt M8x25 | 815 | 2 |
| 7 | Washer M8 | 800 | 2 |
| 8 | Dural. adapter SLD | 537 | 1 |
| 9 | Spring washer M10 | 801 | 1 |
| 10 | Stainless steel rod 12(1,25)x260x30x30 |  | 1 |

| ID | Description | Cat.No | Pcs |
| --- | --- | --- | --- |
| 1 | Galvanized steel nut | X117 | 1 |
| 2 | Bolt M10x110x1,25 | 928 | 1 |
| 3 | Dural. Sleeve l=22mm | 1300 | 1 |
| 4 | Dural. sleeve l=4mm | 1260 | 1 |
| 5 | Dural. sleeve l=70mm | 15 | 1 |
| 6 | Bolt M8x25 | 815 | 2 |
| 7 | Washer M8 | 800 | 2 |
| 8 | Dural. adapter SLD | 537 | 1 |
| 9 | Spring washer M10 | 801 | 1 |

For the left and right side connection, use stainless steel rod ID 10 (length 260mm, threads M12x1,25 mm at the both sides) that is included in the mounting kit

Mounting:

1. Remove the original Bolt M12 from the frame.

2. Insert the stainless steel rod ID 10 into the frame.

3. Insert the sleeve ID 4 from the left and right side. Screw in the galvanized nut ID 1 (tighten against each other) and fit the sleeve ID 3

4. Place the washer M10 ID 9, Dural. adapter SLD ID8 and dural. sleeve ID 5 on the bolt M10 ID 2

5. Screw this set through the sleeve ID 3 into the galvanized nut ID 1 in the motorcycle frame

8. Tighten both sides.

9. Install slider using bolt M8 ID 6 and washer M8 ID 7

Warning! We recommend using medium strength thread glue (blue) during assembly.

4

1

6

3

2

10

5

7

8

9