D20S

Ducati Multistrada 1260S 2018-

Mounting Kit – Left Side:		     		Mounting Kit – Right Side:

| ID | Description | Cat.No | Pcs |
| --- | --- | --- | --- |
| 1 | Galvanized steel nut | X117 | 1 |
| 2 | Bolt M10x100x1,25 | 892 | 1 |
| 3 | Dural. sleeve l=22mm | 1300 | 1 |
| 4 | Dural. sleeve l=4mm | 1260 | 1 |
| 5 | Dural. sleeve l=50mm | 5 | 1 |
| 6 | Dural. Sl adapter l=20mm | 1137 | 1 |
| 7 | Bolt M6x25 | 877 | 3 |
| 8 | Bolt M4x16 | 876 | 2 |
| 9 | Stainless steel rod 12(1,25)x260x30x30 |  | 1 |

| ID | Description | Cat.No | Pcs |
| --- | --- | --- | --- |
| 1 | Galvanized steel nut | X117 | 1 |
| 2 | Bolt M10x105x1,25 | 892 | 1 |
| 3 | Dural. sleeve l=22mm | 1300 | 1 |
| 4 | Dural. sleeve l=4mm | 1260 | 1 |
| 5 | Dural. sleeve l=50mm | 5 | 1 |
| 6 | Dural. Sl adaptér d=20mm | 1137 | 1 |
| 7 | Bolt M6x25 | 877 | 3 |
| 8 | Bolt M4x16 | 876 | 2 |

For the left and right side connection, use stainless steel rod ID 9 (length 260mm, threads M12x1,25 mm at the bothsides) that is included in the mounting kit.

Mounting:

1. Remove the original Bolt M12 from the frame.

2. Insert the stainless steel rod ID 9 into the frame.

3. Insert the sleeve ID 4 from the left and right side. Screw in the galvanized nut ID 1 (tighten against each other) and fit the sleeve ID 3

4. Place the steel insert ID 6 and sleeve ID 5 on the bolt M10 ID 2.

5. Screw this set through the sleeve ID 3 into the galvanized nut ID 1 in the motorcycle frame

8. Tighten both sides

9. Install slider using Bolt ID 7 and Dural plate using bolt ID 8

Warning! We recommend using medium strength thread glue (blue) during assembly.

4

4

1

1

6

2

3

3

2

6

7

5

5

7

8

7

8