D2SD

Ducati Monster 600/625/695/750/800/900

900S/S2R/S4/S4R/S1000/S4RS

2001-

| Mounting Kit - Left Side: Mounting Kit - Right Side: ID | Description | Cat.No | Pcs |
| --- | --- | --- | --- |
| 1 | SLD adapter | 537 | 1 |
| 2 | Lock nut M12 | 811 | 1 |
| 3 | Dural. sleeve l=50mm | 286 | 1 |
| 4 | Bolt M8x25 | 815 | 2 |
| 5 | Washer M8 | 800 | 2 |
| 6 | Dural. sleev l=50mm | 208 | 1 |
| 7 | Stainles steel rod M12x470x30x30 |  | 1 |

| ID | Description | Cat.No | Pcs |
| --- | --- | --- | --- |
| 1 | SLD adapter | 537 | 1 |
| 2 | Lock nut M12 | 811 | 1 |
| 3 | Dural. sleeve l=50mm | 286 | 1 |
| 4 | Bolt M8x25 | 815 | 2 |
| 5 | Washer M8 | 800 | 2 |
| 6 | Dural. sleeve l=50mm | 208 | 1 |

Mounting instructions:

- Remove original bolt M12 from the frame.

- Insert the rod M12x470 ID.7 through the frame.

- Place the dural. sleeves ID.3 on the both sides of the rod.- wider on the right with the sleeves ID.6.

- Install the SLD adapter ID.1 and tighten with nuts M12 ID.2.

- Finally, fasten the slider using bolts M8 ID.4. and washers M8 ID.5

Warning: We recommend using medium strength (blue) thread glue during assembly.