D2S

Ducati Monster 600/625/695/750/800/900

900S/S2R/S4/S4R/S1000/S4RS

2001-

| Left side mounting kit: Right side mounting kit: ID. | Description | Cat.no. | Pcs |
| --- | --- | --- | --- |
| 1 | Steel nut M12 | X12 | 1 |
| 2 | Dural. adapter | 340 | 1 |
| 3 | Dural. cylinder - 15mm | 1148 | 1 |
| 4 | Bolt M6x25 | 877 | 3 |
| 5 | Bolt M4x16 | 876 | 2 |
| 6 | Rod 12(1,25)x378x30x30 | - | 1 |

| ID. | Description | Cat.no. | pcs |
| --- | --- | --- | --- |
| 1 | Steel nut M12 | X12 | 1 |
| 2 | Dural adapter | 340 | 1 |
| 3 | Bolt M6x25 | 877 | 3 |
| 4 | Bolt M4x16 | 876 | 2 |

Instructions:

- Remove original bolt M12 from the frame of motorcycle.

- Insert rod ID6 through the frame.

- Put cylinder ID3 and adapter ID2 on the left side of the rod.

- Put only adapter ID2 on the right side of the rod.

- Mount slider on the adapter using three bolts M6.

- In the end fit coloured cover using two bolts M4.

We recommend to use the glue on all threads (medium strong).