D4

Ducati Monster 696 2008-

Ducati Monster 796 2010-

Ducati Monster 1100/1100S 2009-

| Kit for the left side |  |  |  |  | Kit for the right side |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. | Description | Cat.no. | Pcs |  |  | No. | Description | Cat.no. | Pcs |  |
| 1 | Steel nut M12 | X12/X2 | 1 |  |  | 1 | Steel nut M12 | X12/X2 | 1 |  |
| 2 | Duralumin cylinder d=65mm | 41 | 1 |  |  | 2 | Duralumin cylinder d=65mm | 41 | 1 |  |
|  | Stainless steel bar M12x400x1,25mm | N16 | 1 |  |  |  |  |  |  |  |

Installation:

- dismount the long original bolt M12

- insert through bar the stainless steel bar M12x400x1,25mm

- give duralumin cylinder (No. 2) on this stainless steel bar both sides

- finally screw up all with nuts M12 (No. 1)

Warning – we recommend to use safety glue on threads of all bolts (moderate strong – blue colour).